Analog Lab 5:

Darlington Circuits

Date: 2023/04/27

Class: 電機二全英班

Group: Group 8

B103105006 胡庭翊

B103105018 劉姵妤

Submitter:

B103105006 胡庭翊

- Working Project #1: Photoelectric controller

- Measurement Data

- By connecting the CdS to the multimeter, and let it face to the light, we can measure and get the resistance of 3.49kΩ.

- Using finger to cover the resistor, we measured and got the resistance of 18.30kΩ.

Q1:	Using finger to cover the CdS, the LED light up.

Q2:	Removed finger, the LED turned down.

- Simulation Result

- Adjust the resistor right until the LED turned off. The edge resistance is 3.368kΩ.

- Use finger to cover up the CdS, which is, adjust the R5 from the measured value (uncovered: 3.49kΩ to covered: 18.30kΩ), we found the current passing through LED increased and hence the LED lighted up.

- The LED turned off (current passing through LED dramatically decreased) again after adjusting the resistance from 18.30kΩ back to 3.49Ω, which as, removing the finger.

- Observation and discussion

Observation:

Discussion:

Q:	Why in simulation, the input voltage is 12.7V instead of 12V?

A:	Since the trigger voltage of the SW is 12V, we measure the voltage across SW1, which is V (N001, N002) and adjust the input voltage from 12V to 12.7V, so as the input voltage into SW1 is 12.026V≅12V, which is the very close voltage that can trigger the SW1 to work, and as the result, it gives the current passing through LED 11.968mA.

- Working Project #2:Time Delay Relay

- Measurement Data

Q1:	After turning on the switch for a while, the relay holds and hence the LED lights up.

Q2:	After turning off the switch, turn the potentiometer clockwise to the end, and then turned on the switch again, the relay holds for 0 second to light up, which means, we observed that it re-lighted up immediately.

Q3:	After turning off the switch, turn the potentiometer counterclockwise to the end, and then turned on the switch again, the relay holds for 15 second to light up.

Q4:	Hence we know that the relay can holds at least 0 second and at most 15 seconds.

- Simulation Result

Q1:	After turning on the switch for a while, the relay holds and hence the LED lights up.

The voltage across the diode is V (N001, N002), and the current passing through the diode is I (D1).

Also, V (n001) is the input step voltage of 13V, and V (n008) is the voltage across SW1.

Q2:	After turning off the switch, turn the potentiometer clockwise to the end, and then turned on the switch again, the relay holds for 0 second to light up, which means, we observed that it re-lighted up immediately.

V (n001) is the input step voltage of 13V with 1 second delay.

V (n008) is the voltage across SW1, and I (D1) is the current passing through the diode.

Q3:	After turning off the switch, turn the potentiometer counterclockwise to the end, and then turned on the switch again, the relay holds for 30 seconds to light up.

V (n001) is the input step voltage of 13V with 1 second delay.

V (n008) is the voltage across SW1, and I (D1) is the current passing through the diode.

Q4:	Hence we know that the relay can holds at least 0 second and at most 30 seconds.

- Observations and Discussions

Observation:

Discussion:

Q:	Why in simulation, the input voltage is 13V instead of 12V?

A:	If we input 12V, the capacitor cannot actually be charged to and give the required trigger voltage 12V to SC1, hence the time delay system cannot function.

I (D1) is extremely small and the voltage across SC1 cannot meet its trigger voltage.

Q:	Why the delay time in simulation differs from practical statistic?

A:	The delay time depends on the speed of voltage charge from capacitor, if we given a smaller then 13V but larger then 12V voltage, the delay time, which is the time for voltage across SW1 increased to its trigger voltage, extended.

Q:	What may also be the factor that can affect the delay time?

A:	In the stage of practical measurement, we found that the delay time is extremely short (1 second of delay only). By checking the circuit and components, we found that the β of one of the BJT is un-normally small, which leads to the result that the capacitor charged fast and hence make the delay time short.

- Practice Problems

- Reflection